System Interface Definitions

| Interface | Description |
| --- | --- |
| PWR_IN | 120VAC at 60Hz supply power to entire system |
| Arduino_Power_In | 5V (currently powered by Laptop) for powering Arduino |
| Relay_Power | 5V Power Supply from Nano to Relay Module |
| CH1_Main_Relay | Output high/low to channel 1 whether command should be on/off, respectively |
| CH2_Main_Relay | Output high/low to channel 2 whether command should be on/off, respectively |
| DIM_CS1 | Frequency based pulse train to dimmer channel 1 determined by input from app |
| DIM_CS2 | Frequency based pulse train to dimmer channel 2 determined by input from app |
| IS_1 | *Waiting on parts* Current Sensing for channel 1 of output to produce power consumption data to app |
| IS_2 | *Waiting on parts* Current Sensing for channel 2 of output to produce power consumption data to app |
| Bluetooth_CMD_In | Asynchronous (9600 BAUD) transmission to Bluetooth module of command inputs |
| Bluetooth_CMD_Out | 2 to 5 V output from Arduino Nano powering LED |
| PC_Data | Power consumption data to be transmitted and received between Arduino and Bluetooth app |
| DCPower_USB_In | 12V input from the AC-DC Converter |
| DCPower_USB_Out | 5V at 1A outputted via the dual USB A port (1A when charging one device, 500mA each when charging two devices) |
| Light_Power_In | Following the fuse/switching block, this allows a maximum of 5A to the relay module for full supply amount |
| Converter_In | 120VAC input to be converted to DC output |
| Converter_Out | 12V DC output to Arduino and USB charger |
| Dim_Power_CH1_In | Supplying power to channel 1 on dimmer to be manipulated based DIM_CS1 |
| Dim_Power_CH2_In | Supplying power to channel 2 on dimmer to be manipulated based DIM_CS2 |
| ZCR_Power_In | Power tapped off Light_Power_In to read the zero cross reading of the 120VAC line to determine where DIM_CS1/DIM_CS2 pulses should line up to manipulate Dim_Power_CH1_In/Dim_Power_CH2_In, respectively. |
| CH1_Output_Dim | Direct output of current wave of channel 1 to outlet box. |
| CH2_Output_Dim | Direct output of current wave of channel 2 to outlet box. |