# Final Project: Remote AC Switch

## System Requirements

- Customer Requirement: The switch needs to work for household lights.

Engineering Requirement: The system will turn off and on at least 2 independent household lamps with up to 100W incandescent bulbs and report current watts delivered by each channel.

- Customer Requirement: The system needs to have a simple application
Engineering Requirement: 9 out of 10 users will be able to turn the switches off and on from a mobile phone in less than 10 seconds without any training or having previously seen the interface.

- Customer Requirement: The system must be safe.
Engineering Requirement: The system will use only US standard plugins for connecting to external devices and will not allow any object with a diameter greater than 1mm to enter the enclosure, and will be disable if more than 5A is drawn from the wall power.

- Customer Requirement: The system must have a timer

Engineering Requirement: The system must turn off the output after a time of upto 1 hour ± 1 minute when enabled.

- Customer Requirement: The system must be wireless
Engineering Requirement: The system will be able to accept commands from a mobile phone over 20 feet away from the plugins.

## Additional Requirements

Customer Requirement: The system must include a USB port to charge a device.

Engineering Requirement: The system must include a USB port that can supply 5V and 1A DC to a connected device, to charge that device.

Customer Requirement: The system must include a way to dim the lights through the app.

Engineering Requirement: The system must be able to dim each light independently, within a range of at least 100% to 50% or less brightness, using the app.